Thực hành KTMT Lab 12

Lê Quốc Đảng

20225801

Assignment 1

- Code

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv OUT_ADRESS_HEXA_KEYBOARD 0xFFFF0014

.data

mess: .asciiz "\n"

.text

main:

li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t2, OUT_ADRESS_HEXA_KEYBOARD

li $t3, 0x01 # check row 4 with key 0, 1, 2, 3

li $t4, 0x02 # check row 4 with key 4, 5, 6, 7

li $t5, 0x04 # check row 4 with key 8, 9, A, B

li $t6, 0x08 # check row 4 with key C, D, E, F

polling:

sb $t3, 0($t1) # must reassign expected row

lb $a0, 0($t2) # read scan code of key button

bne $a0, $zero, print

sb $t4, 0($t1) # must reassign expected row

lb $a0, 0($t2) # read scan code of key button

bne $a0, $zero, print

sb $t5, 0($t1) # must reassign expected row

lb $a0, 0($t2) # read scan code of key button

bne $a0, $zero, print

sb $t6, 0($t1) # must reassign expected row

lb $a0, 0($t2) # read scan code of key button

bne $a0, $zero, print

j sleep

print:

li $v0, 34 # print integer (hexa)

syscall

la $a0, mess #print "\n"

li $v0, 4

syscall

sleep:

li $a0, 3000 # sleep 3000ms

li $v0, 32

syscall

back_to_polling:

j polling # continue polling

exit:

- Thực thi

-Chương trình thực hiện lần lượt gán giá trị $t1 lần lượt bằng 0x01, 0x02, 0x04, 0x08 để kiểm tra xem hàng nào được bấm. Load giá trị $t2, nếu  != 0 thì in ra.

-Kết quả

- Bấm phím 0

- Bấm phím 1

- Bấm phím 2

- Và lần lượt như vậy, được kết quả là

Assignment 2

- Code

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv OUT_ADRESS_HEXA_KEYBOARD 0xFFFF0014

.data

Message1: .asciiz "Oh my god. Someone's presed a button: "

Message2: .asciiz "\n"

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# MAIN Procedure

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.text

main:

#---------------------------------------------------------

# Enable interrupts you expect

#---------------------------------------------------------

# Enable the interrupt of Keyboard matrix 4x4 of Digital Lab Sim

li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t7, 0x80 # bit 7 of = 1 to enable interrupt

sb $t7, 0($t1)

#---------------------------------------------------------

# No-end loop, main program, to demo the effective of interrupt

#---------------------------------------------------------

Loop: nop

nop

nop

nop

b Loop # Wait for interrupt

end_main:

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~ ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# GENERAL INTERRUPT SERVED ROUTINE for all interrupts

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.ktext 0x80000180

#--------------------------------------------------------

# Processing

#--------------------------------------------------------

addi $v0, $zero, 4 # show message

la $a0, Message1

syscall

li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t2, OUT_ADRESS_HEXA_KEYBOARD

li $t3, 0x01 # check row 4 with key 0, 1, 2, 3

li $t4, 0x02 # check row 4 with key 4, 5, 6, 7

li $t5, 0x04 # check row 4 with key 8, 9, A, B

li $t6, 0x08 # check row 4 with key C, D, E, F

sb $t3, 0($t1) # must reassign expected row

lbu $a0, 0($t2) # read scan code of key button

bne $a0, $zero, print

sb $t4, 0($t1) # must reassign expected row

lbu $a0, 0($t2) # read scan code of key button

bne $a0, $zero, print

sb $t5, 0($t1) # must reassign expected row

lbu $a0, 0($t2) # read scan code of key button

bne $a0, $zero, print

sb $t6, 0($t1) # must reassign expected row

lbu $a0, 0($t2) # read scan code of key button

bne $a0, $zero, print

print:

li $v0, 34 # print integer (hexa)

syscall

addi $v0, $zero, 4 # show message

la $a0, Message2

syscall

li $t7, 0x80 # bit 7 of = 1 to enable interrupt

sb $t7, 0($t1)

#--------------------------------------------------------

# Evaluate the return address of main routine

# epc <= epc + 4

#--------------------------------------------------------

next_pc:mfc0 $at, $14 # $at <= Coproc0.$14 = Coproc0.epc

addi $at, $at, 4 # $at = $at + 4 (next instruction)

mtc0 $at, $14 # Coproc0.$14 = Coproc0.epc <= $at

return: eret # Return from exception

- Thực thi

-Chương trình thực hiện kiểm tra phím được bấm tương tụ như bài 1. Sau khi in ra kí tự được bấm thì gán giá trị ở IN_ADRESS_HEXA_KEYBOARD = 0x80 để enable interrupt.

-Bấm phím 0

-Bấm phím 9

-Bấm phím f

Assignment 3

- Code

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv OUT_ADRESS_HEXA_KEYBOARD 0xFFFF0014

.data

Message: .asciiz "Key scan code "

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# MAIN Procedure

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.text

main:

#---------------------------------------------------------

# Enable interrupts you expect

#---------------------------------------------------------

# Enable the interrupt of Keyboard matrix 4x4 of Digital Lab Sim

li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t2, OUT_ADRESS_HEXA_KEYBOARD

li $t3, 0x80 # bit 7 = 1 to enable

sb $t3, 0($t1)

#---------------------------------------------------------

# Loop an print sequence numbers

#---------------------------------------------------------

xor $s0, $s0, $s0 # count = $s0 = 0

Loop: addi $s0, $s0, 1 # count = count + 1

prn_seq:addi $v0,$zero,1

add $a0,$s0,$zero # print auto sequence number

syscall

prn_eol:addi $v0,$zero,11

li $a0,'\n' # print endofline

syscall

sleep: addi $v0,$zero,32

li $a0,1000 # sleep 300 ms

syscall

nop # WARNING: nop is mandatory here.

b Loop # Loop

end_main:

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# GENERAL INTERRUPT SERVED ROUTINE for all interrupts

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.ktext 0x80000180

#-------------------------------------------------------

# SAVE the current REG FILE to stack

#-------------------------------------------------------

IntSR: addi $sp,$sp,4 # Save $ra because we may change it later

sw $ra,0($sp)

addi $sp,$sp,4 # Save $ra because we may change it later

sw $at,0($sp)

addi $sp,$sp,4 # Save $ra because we may change it later

sw $v0,0($sp)

addi $sp,$sp,4 # Save $a0, because we may change it later

sw $a0,0($sp)

addi $sp,$sp,4 # Save $t1, because we may change it later

sw $t1,0($sp)

addi $sp,$sp,4 # Save $t2, because we may change it later

sw $t2,0($sp)

addi $sp,$sp,4 # Save $t3, because we may change it later

sw $t3,0($sp)

addi $sp,$sp,4 # Save $t4, because we may change it later

sw $t4,0($sp)

addi $sp,$sp,4 # Save $t5, because we may change it later

sw $t5,0($sp)

addi $sp,$sp,4 # Save $t6, because we may change it later

sw $t6,0($sp)

#--------------------------------------------------------

# Processing

#--------------------------------------------------------

prn_msg: addi $v0, $zero, 4

la $a0, Message

syscall

get_cod: li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t2, OUT_ADRESS_HEXA_KEYBOARD

li $t3, 0x01 # check row 1 with key 0, 1, 2, 3

li $t4, 0x02 # check row 2 with key 4, 5, 6, 7

li $t5, 0x04 # check row 3 with key 8, 9, A, B

li $t6, 0x08 # check row 4 with key C, D, E, F

sb $t3, 0($t1 ) # must reassign expected row

lbu $a0, 0($t2) # read scan code of key button

bne $a0, $zero, prn_cod

sb $t4, 0($t1 ) # must reassign expected row

lbu $a0, 0($t2) # read scan code of key button

bne $a0, $zero, prn_cod

sb $t5, 0($t1 ) # must reassign expected row

lbu $a0, 0($t2) # read scan code of key button

bne $a0, $zero, prn_cod

sb $t6, 0($t1 ) # must reassign expected row

lbu $a0, 0($t2) # read scan code of key button

bne $a0, $zero, prn_cod

prn_cod:li $v0,34

syscall

li $v0,11

li $a0,'\n' # print endofline

syscall

#--------------------------------------------------------

# Evaluate the return address of main routine

# epc <= epc + 4

#--------------------------------------------------------

next_pc:mfc0 $at, $14 # $at <= Coproc0.$14 = Coproc0.epc

addi $at, $at, 4 # $at = $at + 4 (next instruction)

mtc0 $at, $14 # Coproc0.$14 = Coproc0.epc <= $at

#--------------------------------------------------------

# RESTORE the REG FILE from STACK

#--------------------------------------------------------

restore: li $t7, 0x80 # bit 7 of = 1 to enable interrupt

sb $t7, 0($t1)

lw $t6, 0($sp) # Restore the registers from stack

addi $sp,$sp,-4

lw $t5, 0($sp) # Restore the registers from stack

addi $sp,$sp,-4

lw $t4, 0($sp) # Restore the registers from stack

addi $sp,$sp,-4

lw $t3, 0($sp) # Restore the registers from stack

addi $sp,$sp,-4

lw $t2, 0($sp) # Restore the registers from stack

addi $sp,$sp,-4

lw $t1, 0($sp) # Restore the registers from stack

addi $sp,$sp,-4

lw $a0, 0($sp) # Restore the registers from stack

addi $sp,$sp,-4

lw $v0, 0($sp) # Restore the registers from stack

addi $sp,$sp,-4

lw $at, 0($sp) # Restore the registers from stack

addi $sp,$sp,-4

lw $ra, 0($sp) # Restore the registers from stack

addi $sp,$sp,-4

return: eret # Return from exception

- Thực thi

-Chương trình kiểm tra nút được bấm tương tự như bài 2. Sau khi in ra kí tự được bấm thì gán giá trị ở IN_ADRESS_HEXA_KEYBOARD = 0x80 để enable interrupt.

-Bấm phím 1

-Bấm phím a

-Bấm phím d

-Bấm phím c