Thực hành KTMT Lab 13

Lê Quốc Đảng

20225801

Assignment 4

- Code

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv OUT_ADRESS_HEXA_KEYBOARD 0xFFFF0014

.eqv COUNTER 0xFFFF0013 # Time Counter

.eqv MASK_CAUSE_COUNTER 0x00000400 # Bit 10: Counter interrupt

.eqv MASK_CAUSE_KEYMATRIX 0x00000800 # Bit 11: Key matrixinterrupt

.data

msg_keypress: .asciiz "Someone has pressed a key: "

msg: .asciiz "\n"

msg_counter: .asciiz "Time inteval!\n"

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# MAIN Procedure

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.text

main:

#---------------------------------------------------------

# Enable interrupts you expect

#---------------------------------------------------------

# Enable the interrupt of Keyboard matrix 4x4 of Digital Lab Sim

li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t3, 0x80 # bit 7 = 1 to enable

sb $t3, 0($t1)

# Enable the interrupt of TimeCounter of Digital Lab Sim

li $t1, COUNTER

sb $t1, 0($t1)

#---------------------------------------------------------

# Loop an print sequence numbers

#---------------------------------------------------------

Loop: nop

nop

nop

sleep: addi $v0,$zero,32 # BUG: must sleep to wait for Time Counter

li $a0,200 # sleep 300 ms

syscall

nop # WARNING: nop is mandatory here.

b Loop

end_main:

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# GENERAL INTERRUPT SERVED ROUTINE for all interrupts

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.ktext 0x80000180

IntSR: #--------------------------------------------------------

# Temporary disable interrupt

#--------------------------------------------------------

dis_int:li $t1, COUNTER # BUG: must disable with Time Counter

sb $zero, 0($t1)

# no need to disable keyboard matrix interrupt

#--------------------------------------------------------

# Processing

#--------------------------------------------------------

get_caus:mfc0 $t1, $13 # $t1 = Coproc0.cause

IsCount:li $t2, MASK_CAUSE_COUNTER# if Cause value confirm Counter..

and $at, $t1,$t2

beq $at,$t2, Counter_Intr

IsKeyMa:li $t2, MASK_CAUSE_KEYMATRIX # if Cause value confirm Key..

and $at, $t1,$t2

beq $at,$t2, Keymatrix_Intr

others: j end_process # other cases

Keymatrix_Intr: li $v0, 4 # Processing Key Matrix Interrupt

la $a0, msg_keypress

syscall

#--------------------------------------------------------

# Kiểm tra xem nút nào được bấm

li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t2, OUT_ADRESS_HEXA_KEYBOARD

li $t3, 0x01 # check row 4 with key 0, 1, 2, 3

li $t4, 0x02 # check row 4 with key 4, 5, 6, 7

li $t5, 0x04 # check row 4 with key 8, 9, A, B

li $t6, 0x08 # check row 4 with key C, D, E, F

sb $t3, 0($t1) # must reassign expected row

lbu $a0, 0($t2) # read scan code of key button

bne $a0, $zero, print

sb $t4, 0($t1) # must reassign expected row

lbu $a0, 0($t2) # read scan code of key button

bne $a0, $zero, print

sb $t5, 0($t1) # must reassign expected row

lbu $a0, 0($t2) # read scan code of key button

bne $a0, $zero, print

sb $t6, 0($t1) # must reassign expected row

lbu $a0, 0($t2) # read scan code of key button

bne $a0, $zero, print

#--------------------------------------------------------

print:

li $v0, 34 # print integer (hexa)

syscall

print_n:

li $v0, 4 # Processing Counter Interrupt

la $a0, msg

syscall

#--------------------------------------------------------

# Re-enable the interrupt of Keyboard matrix 4x4 of Digital Lab Sim

li $t2, IN_ADRESS_HEXA_KEYBOARD

li $t3, 0x80 # bit 7 = 1 to enable

sb $t3, 0($t2)

#--------------------------------------------------------

j end_process

Counter_Intr: li $v0, 4 # Processing Counter Interrupt

la $a0, msg_counter

syscall

j end_process

end_process:

mtc0 $zero, $13 # Must clear cause reg

en_int: #--------------------------------------------------------

# Re-enable interrupt

#--------------------------------------------------------

li $t1, COUNTER

sb $t1, 0($t1)

#--------------------------------------------------------

# Evaluate the return address of main routine

# epc <= epc + 4

#--------------------------------------------------------

next_pc:mfc0 $at, $14 # $at <= Coproc0.$14 = Coproc0.epc

addi $at, $at, 4 # $at = $at + 4 (next instruction)

mtc0 $at, $14 # Coproc0.$14 = Coproc0.epc <= $at

return: eret # Return from exception

- Thực thi

-Khi nhảy đoạn mã cho xử lí ngắt, kiểm tra xem đó là ngắt của Counter hay Keyboard matrix 4x4.

-Nếu là của Keyboard kiểm tra xem là bấm nút nào bằng cách lần lượt gán giá trị $t1 lần lượt bằng 0x01, 0x02, 0x04, 0x08 để kiểm tra xem hàng nào được bấm. Load giá trị $t2, nếu  != 0 thì in ra. Sau khi in ra thì re-able ngắt của Keyboard.

-Kết quả

Bấm nút 4

Bấm nút 9

Bấm nút 6

Bấm nút 1

Assignment 5

- Code

.eqv KEY_CODE 0xFFFF0004 # ASCII code from keyboard, 1 byte

.eqv KEY_READY 0xFFFF0000 # =1 if has a new keycode ?

# Auto clear after lw

.eqv DISPLAY_CODE 0xFFFF000C # ASCII code to show, 1 byte

.eqv DISPLAY_READY 0xFFFF0008 # =1 if the display has already to do

# Auto clear after sw

.eqv MASK_CAUSE_KEYBOARD 0x0000034 # Keyboard Cause

.text

li $k0, KEY_CODE

li $k1, KEY_READY

li $s0, DISPLAY_CODE

li $s1, DISPLAY_READY

loop: nop

WaitForKey: lw $t1, 0($k1) # $t1 = [$k1] = KEY_READY

beq $t1, $zero, WaitForKey # if $t1 == 0 then Polling

MakeIntR: teqi $t1, 1 # if $t1 = 1 then raise an Interrupt

j loop

#---------------------------------------------------------------

# Interrupt subroutine

#---------------------------------------------------------------

.ktext 0x80000180

get_caus: mfc0 $t1, $13 # $t1 = Coproc0.cause

IsCount: li $t2, MASK_CAUSE_KEYBOARD# if Cause value confirm Keyboard..

and $at, $t1,$t2

beq $at,$t2, Counter_Keyboard

j end_process

Counter_Keyboard:

ReadKey: lw $t0, 0($k0) # $t0 = [$k0] = KEY_CODE

WaitForDis: lw $t2, 0($s1) # $t2 = [$s1] = DISPLAY_READY

beq $t2, $zero, WaitForDis # if $t2 == 0 then Polling

Encrypt: addi $t0, $t0, 0 # change input key

ShowKey: sw $t0, 0($s0) # show key

nop

end_process:

next_pc: mfc0 $at, $14 # $at <= Coproc0.$14 = Coproc0.epc

addi $at, $at, 4 # $at = $at + 4 (next instruction)

mtc0 $at, $14 # Coproc0.$14 = Coproc0.epc <= $at

return: eret # Return from exception

- Thực thi

-Chương trình thực hiện kiểm tra nếu bàn phím được bấm thì nhảy vào đoạn mã cho xử lí ngắt bằng lệnh teqi.

-Ở đoạn mã xử lí ngắt, kiểm tra xem có phải ngắt của bàn phím hay không. Nếu phải thì show key đó ra màn hình.

-Kết quả